WHP Racer 5G CQA1 检验作业指导书

| Files making and revising resume 文件制/修订履历表 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Maker/Reviser 制/修订人 | Making and revision content 制/修订内容 | Pages 影响页次 | Audit 审核 | Approve 批准 | Version 版本 | Issuing date 发行日期 |
| 赵力成 | WHP Racer 5G CQA1 检验作业指导书 | ALL | DCC | 王瑞飞 | V1.0 | 2020/4/17 |

文件自发布之日起生效执行

文件使用者确保在使用前获取为最新版本

本文件最新版本保存在DPMS

打印版本仅供参考

对流程有修改意见的使用者，应通过邮件发送给文件负责人

此文件为公司DPMS系统保密资产，请妥善保管

- Propose/目的

定义及规范联想武汉厂Racer 5G CQA检验项目及方法,作为CQA检验人员的依据。

Clarify Racer 5G CQA checking items and contents in Lenovo Wuhan factory.

2.Scope/适用范围

适用于武汉厂Racer 5G QA人员

Document for Racer 5G QA inspectors in Lenovo Wuhan factory

3.Roles & Responsibilities/权责

PQE负责本标准在武汉厂的更新和发布

PQE is responsible for updating and releasing this document.

4.Definitions/定义：

| 表面等级 分类 Surface classification | - “AA” 级面: 只包括信息显示区域以及前后摄像头成像区域和以及前后摄像头非成像区域 Class “AA” surface: Information display area only and Camera View Area& Non View Area. - “A” 级面: 暴露在外，正常使用中直接可见的主要表面(摄像头塑料区域)。 Class “A” surface: Primary surface exposed to direct close-up view in ordinary use and Front and rear cameras can't imaging - “B” 级面: 正常使用时，不能直接可见，暴露在外的表面。 Class “B” surface: Secondary surface exposed but not in direct view in ordinary use. - “C” 级面: 正常使用不可见的表面 Class “C” surface: Surface not visible in ordinary use. - “D” 级面:不可见的表面，必须满足防护和功能的要求。 Class “D” surface: Non appearance surface, must meet protective and functional requirements. 6.详细定义如下 Detail Definition as below: Surface Class/表面等级. Class AA; Class A; Class B; Class C. Front Camera Lens 前摄像头玻璃; AA. Display Lens View Area 屏幕显示区域; AA. Display Lens Non View Area 屏幕非显示区域; A. Rear camera Lens view Area 后拍镜片显示区域; AA. Rear camera Lens Non view Area 后拍镜片非显示区域; A. Rear Flash Lens 闪光灯镜片; A. Mid Frame Housing 壳体中框; A. Inside SIM DOOR Opening 卡托内部开放区域; B. SIM DOOR 卡托; A. Rear Camera Bezel 后拍装饰件; A. Inside SIM DOOR Opening 卡托内部开放区域; A. Glass Inlay 镶嵌物表面; B. Volume & Power Buttons 音量键&电源键; A. |
| --- | --- |

- Content/作业内容

| Cosmetic and Accessory 外观和辅料 |  |  |
| --- | --- | --- |
| No | 外观检验 Cosmetic inspection | 标准 Standard |
| 1 | 屏幕/后盖 | - 检查屏上是否有划痕、气泡和不可移除的污迹 - 检查屏幕四周是否翘起，夹胶、偏移、松动 - 检验屏幕与中框上下左右间隙、防尘网与中框间隙是否OK - 尤其注意检验Lens、边框胶爬墙以及显示屏水纹、凸点异常、橘皮 - 注意听筒处是否有异物、堵孔\破损。 - 边缘无溢胶、漏铜等（除非有样本）。 |
| 2 | 中框 Mid housing | - 检查前后盖组装要匹配,颜色要匹配,安装要到位。 - 检查外壳无损伤、划痕、印刷污迹、不可移除的污迹、无掉漆。 - 注意Mic孔有无堵孔异常。 1. Check the cover before and after assembly to match, the color to match, the installation should be in place. 2. Review the shell no damage, scratches, printing stains, do not remove the stains, no off paint. 3. Pay attention to Mic2 abnormal without blocking. |
| 3 | 开机键/音量键/卡托 Powerbutton/volume button/Sim tray | - 检查按键颜色与料号要匹配。逐个按侧键，弹性无异常，均匀度一致且组装到位。 Check the button color match with the material number.Press side keys one by one, receive tactile response and be better assembled. - 检查SIM单双卡信息与主机头对应正确。 Checking the SIM single /double SIM Card is match with the xcvr - 需要特别注意按键周边是否有漏白。 Special attention needs to be paid to whether there is any leakage around the keys. |
| 4 | 插口 Connector hole | 孔内无异物，无损伤，无不可移除的污迹。 No foreign body in the hole, no damage, no not to remove stains. |
| 5 | 照相头/闪光灯 Camera head/flash | 检查摄像头是否歪斜，不能有损伤，不可移除的污迹异物等。 检查闪光灯柱无缺损，不可移除的污迹异物等。 1. Check whether the camera is skew, there can be no injury, shall not remove the stain from foreign bodies, etc. 2. Check the flash lamp without defect, and not to remove stains foreign bodies, etc. |
| 6 | 标签 Label | 1. 检查label是否丢失，印刷是否清楚、无断码。 2. CQA模式下的barcode与Label Tracking ID一致；主机物料编号与MES一致。 1. Check whether the label is missing, the printing is clear, no break code. 2. CQA mode is consistent with barcode Label Tracking ID.The XCVA number consistent with MES. |
| 7 | 其它外观标准 Other Cosmetic spec | 参照《MBG-S-PQM-061外观检验标准》、《SMT外观检验标准》 Reference [061] handheld product appearance standard (MOTOROLA), "the SMT appearance inspection standard" |
| 10 | 断差与缝隙标准 Step or gap spec | 参照PSCD管控标准：详见《产品外观检验规范标准和品质保证》 Consult standard PSCD control: see "product appearance inspection standard and quality guarantee" |
| 11 | 拉拔测试 Pull test | TP/inlay拉拔测试2Pcs/day - CQA（按照首件检验要求进行测试） - 20N/10S，并记录在《WHM-QA-QR-136 WHP通用拉拔测试记录表》 Follow the FAI/2Pcs test by MO, and waiting for the size to check list. |
| No | 功能检验 Function test | 标准 Standard |
| 11 | 检前须知 Inspection instructions before | 检验前针对按上下音量键进入下一项测试，请注意以下几点： 1. 按下音量键：该测试项Pass 2. 按上音量键：该测试项Fail 3. 在按上、下音量键进入下一项测试之前，会有短暂的停顿状态，不要长按，否则会进入下下测试，并且Pass 或Fail了上一项测试，必须等待。 4. 通常只需短按上、下音量键进入下一项测试。但在Key Test和Headset Info test中，必须长按不超过2秒钟进入下一项测试，因为短按只是在测试按键功能。 5. 在Sequential Test测完后，屏幕上回显示绿色英文的测试结果列表。 6. 插入SIM卡时如有阻塞感，切勿强行插入避免造成卡槽损坏。 1. Press the volume button: the test Pass 2. Click on the volume button: the test Fail 3. In the upper and lower the volume button to enter under a test before, there will be a brief pause state, not to long press, otherwise it will enter down test, and Pass or Fail in a test, have to wait. 4. Usually just short on upper and lower the volume button to enter the next test.But in the Key Test and Headset Info in the Test, must be long as no more than 2 seconds into the next Test, short press just Test the Key function. 5. In the Sequential Test after the Test, the screen shows green English Test result list last time. 6. Insert SIM card, do not forcibly insert avoid causing damage of card slot. |
| 12 | 插入NanoSIM卡开机 Insert mini SIMand Turn on unit | 1. 拔出SIM卡托插入Nano SIM卡。 2. 按电源键开机,有轻微震感。 3. 拨打112，使用手指遮挡P-sensor区域确认手机是否会进入黑屏状态 1. Pull out the SIM Tray, insert the mini SIM card 2. Turn on unit with the power key with light vibration. 3. Dial 112 and use your finger to block the p-sensor area to confirm whether the phone will go into black |
| 13 | CQA 测试 CQA Test | 进入CQA测试模式：点击手机屏幕右下角“文件夹”图标，点击拨号图标（Phone），输入“*#*#2486#*#*”, 选Sequence Mode，弹出Confirmation对话框中选择“YES”，出现任何弹窗提示点击“Allow”进入测试： 1.Touch Folder in lower right corner，Select CQATest ，If CQA icon is not available, follow the steps below ，Select Phone Icon，Select Dialer Icon，In dialer screen enter *#*#2486#*#*；When CQA is started for first time, there will be permissions that need to be allowed，Click “allow” on the Pictures and Videos screen，Click “allow” on the Record Audio screen，Click “allow” on the Location screen，Click “allow” on the Manage Phone Calls screen，Click “allow” on the Photos Media and Files screen，Click “allow” on the Sensor Data Vital Sign screen. - Headset/Headset Info: 显示 - Headset Detect Status: UNPLUGGED. - Headset Has Microphone: NO. - Headset Key Press Status: NA； - 插耳机变为: - Headset Detect Status: PLUGGED. - Has Microphone : YES， - Headset Key Press Status: NA. 2）按耳机开关显示KEYCODE_ HEASSET HOOK pressed, 松开显示released； 拔下耳机显示如下:Headset/Headset Info: 显示 - Headset Detect Status: UNPLUGGED. - Headset Type: No Device. - Headset Has Microphone: No. - Headset Key Press Status: KEYCODE_ HEADSET HOOK -released 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。（注：需长按下音量键，才可进入下一个测试项，且最长只能按2秒钟） If the test FAIL, press on the volume FAIL;If PASS, then press the volume button to enter the next test.(note: need to long press the volume button, to get to the next test, and the longest only by 2 seconds) - NFC test：将测试白卡放在手机背面(转轴位置)，NFC test 界面出现左图则OK, 出现右图时NG. 如测试NG可调整卡片放置位置在测试。多次测试NG则测试Fail。 Put test card to touch the back of phone（hinge area）, NFC test result as left picture, it’s OK, it’s NG if as right picture. If test fail, try to touch card many times, NG also it’s true failure. NG OK - Key/Key Test：显示No Key is pressed. - 按上音量键显示KEYCODE_VOLUME_ UP Pressed, 松开显示KEYCODE_VOLUME _UP Released。 - 按下音量键显示KEYCODE_VOLUME DOWN Pressed,松开显示KEYCODE_VOLUME_ DOWN Released，按下音量键。 - 按电源键，屏幕变暗后返回界面显示KEYCODE_POWER Pressed and Released。 - 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 3.Hit Volume Up key，Verify correct key code response，Hit Volume Down key，Verify correct key code response，Press power key，Verify display goes dark and comes back with correct key code response，Press and Hold Volume Down key for a one second, and then let go to pass and move to the next test，To fail the test, Press and Hold Volume Up key for a one seconds and then let go Volume Down Key Power Key - Bluetooth/ Bluetooth Scan：出现对话框，点击 Allow, 搜索蓝牙设备，若未收索到蓝牙设备，则此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Select ‘Allow’ to turn on Bluetooth，Verify Bluetooth devices are discovered，Hit Volume Down to pass, Volume Up to fail. - Bluetooth/ Bluetooth Mac：显示蓝牙地址，若未出现蓝牙地址，前6位数值信息一致，后两位为变量，则此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。(出现为连续的00也为fail) Verify Bluetooth Mac address is displayed，the first 6 numbers are same, the last two numbers are changeable. Press and Hold Volume Down key to pass and move to the next test，To fail the test, Press and Hold Volume Up key. Note address hasn’t continuous“0”. - Audio/Mic loopback - 显示DEFAULTMIC, 依次点击PRIMARY MIC，SECONDARY MIC、对着手机MIC说“喂喂喂”，能从扬声器中听到清晰的回音，SECONDARY MIC测试会有轻微回音。每个MIC测试完成后用手指堵住MIC孔并分别对着MIC说“喂喂喂”，如果能从扬声器中听到清晰的回音则NG. (Mic1/2使用耳机插入测试) - 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - 注意： MIC 正常功能测试距离：5-10 cm. MIC 漏音测试距离：30cm。 - Press the hole of PRIMARY MIC、SECONDARY MIC、and MIC 4, Put phone to ear and talk into MIC, listen for your voice from ‘earpiece’. And every MIC should be pressed by finger, then put phone to ear and talk into MIC, listen for your voice from ‘earpiece’.(Mic2 Do not insert the headset test once) - HEADSET MIC test: insert headset and HEADSET MIC , put phone to ear and talk into MIC, listen for your voice from ‘earpiece’. - Press VOL DOWN key to pass TEST or VOL UP to fail TEST(‘audio/MIC Loopback’) - Note: 5-10 cm for a voice test, and 30cm for “echo” or seal testing to isolate any mechanical coupling. - Ear speaker： - BUZZ Test：把手机放到耳边，能听到高低交错的长鸣音，检查是否有异常，如声音偏小，有杂音等。注意：测试两次或以上。 - 再选择Play Harvard speed pattern，把手机放到耳边，能听到一段英文对话，检查是否有异常，如声音偏小，有杂音等。 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 7.Listen to BUZZ TEST audio from Ear Speaker,Verify audio is clear,Select PLAY HARVARD SPEECH PATTERN,Listen for media audio from Ear Speaker;Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key,Alert Speaker,Listen to audio file from Alert Speaker,Verify audio is clear;Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - Audio/Loudspeaker： 1) 点击Play Media File –MOTO.OGG, 能听到喇叭音或音乐,且声音清晰无杂音。然后分别选择Left&Right进行测试，如图所示。 2) 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Listen to audio file from Alert Speaker;Verify audio is clear,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - SIM Card: 1）只需检查SIM serial number：SIM卡序列号以及信号图标出现即可，如下图。双卡手机需要测试2张SIM卡。（注：SIM state 可能显示UNKNOWN，视为PASS）。 - 测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify SIM STATE: READY and SIM parameters listed,When using 2 SIM cards, verify that the SIM2 parameters are listed,Press and Hold Volume Down key to pass and move to the next test,To fail the test, Press and Hold Volume Up key - Version版本测试： 1）显示AP Version，BP Version，CQA Test Package Version，RAM与ROM容量与FBR信息对照表一致即可。 2）若不一致，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify AP Version, BP Version, and CQATest Package Version，RAM and Rom are matching what is specified by BR ,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - Display显示屏测试：（主要检查显示屏同色点，异色点，划伤，若有其它缺陷，则视为FAIL，除非有限度样本。） 1）低亮度六色块-->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 2）高亮度六色块-->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 3）从黑到灰渐变色->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 4）黑屏->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 5）白屏->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 6）黑屏白边框。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。特别注意前置摄像头区域。 7）注意：白屏界面，需要将亮度调节至最低检查屏幕是否有异色问题。 Verify ‘Medium Color Bar’ for defects,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key,Verify ‘Bright Color Bar’ for defects and follow step 2) ,Verify ‘Grayscale’ for defects and follow step 2) ,Verify ‘Black’ for defects and follow step 2) ,Verify ‘White with Black Boarder’ for defects and follow step 2) ,Verify ‘Black with White Boarder’ for defects and follow step 2) - Sensor/ Front Light光感测试: 显示绿色英文：Light Sensor：Normal 1) 用手或手指遮挡Light sensor（位于手机顶部）, 显示Light Sensor: Dark’. 2) 手或手指移开light sensor后，显示‘Light Sensor: Bright’。 3）若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify ‘Front Light’ sensor ,Verify that blocking the light sensor (located at the top of the phone) with a hand or finger causes phone to indicate ‘Light Sensor: Dark’, Verify that if light sensor is not blocked phone indicates ‘Light Sensor Brightest, Brighter, Bright, or Normal’,Press and Hold Volume Down key to pass and move to the next test,To fail the test, Press and Hold Volume Up key - Sensor/proximity距离感应器测试： 1) 用手遮到感应点，显示绿色英文“Proximity Sensor: Enable”， 2) 拿开手, 显示红色英文“Proximity Sensor: Disable”后自动进入下一项测试,若不出现Enable或Disable或出现其他异常，此次测试FAIL, 则按上音量键表示Fail；Verify ‘Proximity’ sensor,Verify that with nothing near or touching the proximity sensor (located at top of the phone) the phone indicates ‘Status: Disable’ in Red ,Verify that if proximity sensor is touched or blocked with a finger or hand the phone indicates ‘Status : Enable’ in Green and then ‘Status Saturation’ in Blue,Phone automatically switches back to next menu after Proximity test passes,To fail the test, Press and Hold Volume Up key. - WLAN/Scan Networks:显示网络名称。若未收索到，则此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify WLAN Networks are discovered and Network list appears,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - WLAN:/WiFi Mac：显示网络地址即可。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。(出现为连续的00也为fail) ‘WiFi Mac’test ,Verify WiFi Mac Address is displayed,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - Vibratory马达测试: 按“OFF”有震动，按“ON”震动停止，检查震动是否停止，不能出现震动异音，震动异常等。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。注意：测试三次或以上 Touch On / Off switch to turn vibrator on,Verify vibrator is on,Touch On / Off switch to turn vibrator off,Verify vibrator off,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key - Battery/Battery Info:插上有线或使用无线充电器，检查电池图标里是否有闪电图标，并检查前6项符合如下标准（粗体） - 此项检查在检查完其他项目后单独测试。 - Battery Level>0%， - Battery Voltage >3.000V, - Battery Healthy: BATTERY_HEALTHY_GOOD. - Battery Plugged: BATTERY_ PLUGGED_AC - Battery Present: TRUE（不能是FALSE） 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 注意：电量大于等于15%为PASS。 1.电量＜50%时，屏幕背景色为黑色。 2.50% ≤电量<60%时，屏幕背景色为红色。 3. 60% ≤电量<80%时，屏幕背景色为黄色。 4.电量≥80%时，屏幕背景色为绿色。 Verify ‘BatteryInfo’ - Battery Level should be higher than 18% - Battery Voltage should be higher than 3.700V - Battery Health should indicate BATTERY_HEALTH_UNKNOWN - Battery Plugged:BATTEAY_PLUGGED_AC/BATTEAY_PLUGGED_WIRELESS - Battery Present: TRUE - Battery Status:Battery_STATUS_NOT_CHARGING - Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key NOTE: - Battery Level will be in RED for results < 50% - Background color will change for results >50% - Background is ORANGE for <60% - Background is YELLOW for <80% - Background is GREEN for > 80% - USB Host Mode test /OTG测试: 插入OTG，显示绿色文字，如右图，则PASS. 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Insert OTG, test result is shown as right picture, test pass. Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key - Touch Screen触屏测试：用手指点击白色圆点，沿着白色路径划线，划完后，检查屏幕是否变绿，变绿后将自动进入下一项测试。注意是否有跳线不良现象。 Trace the path with a finger until screen turns green indicating Pass,Test will Pass automatically when tracing is completed successfully,To fail the test, Press and Hold Volume Up key. - Capsense test /电容感应测试 点击capsense 进入测试，将手掌贴近感应器对应位置（CH0/CH2/CH3），状态显示由“absence”转为“presence”,则测试pass，如状态显示“absence ”未变化，则测试fail确认下CAP SENSER。测试位置如图.若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key Press capsense button to enter test , push hands close to sensor which is on the corresponding position ( CH0/CH2). if the status turn form “absence” to “presence”, the test is pass ;if the status of “absence ”can’t change ,the test is fail. Note: - Camera多媒体测试： Front camera: 30cm to 50cm前拍 Rear Main camera 64M: 10cm to 10M.后拍主摄 Rear Tele camera 8M: 35cm to 10M.长焦 Rear WFOV（Wide field-of-view depth mode）camera 16M: 2.5cm to 10m.景深 1 进入后摄像头（Rear1/Rear2/Rear3）取景状态，检查取景是否有异常，如模糊，异色，缺划等，若有异常，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。参照图一。 2 进入后置摄像头（Rear1/Rear2/Rear3）录像状态，点击“start capture”开始录像，对着手机说‘喂喂喂和1234’，最多录10秒后，点击“stop capture”停止录像。点击“start playback”播放录像，查看录像是否清晰，声音是否正常，点击“stop playback”停止播放。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - 进入后摄像头依次（Rear1/Rear2/Rear3）拍照界面，点击屏幕任意一处进行拍照，注意听是否有“咔嚓”一声响（注：若没听到“咔擦”声，请注意音量是否调至最大）且闪光灯闪亮。拍照时，检查是否有异常，如模糊，异色，缺划等，若不能拍照，无咔嚓声或闪光灯不亮或存在其他缺陷，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - 进入前摄像头拍照界面，点击屏幕任意一处进行拍照，注意听是否有“咔嚓”一声响（注：若没听到“咔擦”声，请注意音量是否调至最大，前拍无闪光灯）。拍照时，检查是否有异常，如模糊，异色，缺划等，若不能拍照，无咔嚓声或存在其他缺陷，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 图2 图1 图2 图2 图3 - Verify the rear camera(Rear1/Rear2/Rear3)‘s preview is normal, no contrast color, light line etc. on the screen. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. As picture 1. - Enter rear camera (Rear1/Rear2/Rear3) video interface, press the ‘Start Capture’ button to start video capture; after no more than 10 seconds press the ‘Stop capture’ button; Press the ‘Start Playback’ button and confirm playback works; Press ‘Stop Playback” button and then stops. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. As picture 2. - Enter rear camera(Rear1/Rear2/Rear3) , press anywhere on the screen to take a picture; Listen for the camera “snap shot” sound (no “snap shot” sound，please turn up the VOL UP key) with flash lights.Press VOL DOWN key to pass TEST or VOL UP to fail TEST. As picture 3. - Enter front camera(RGB) , press anywhere on the screen to take a picture; Listen for the camera “snap shot” sound (no “snap shot” sound，please turn up the VOL UP key) , note front camera is without flash lights. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Sensor/Fingerprint Test指纹感应器测试 : SELF TEST. –提示Pass即可 CHECKER BOARD TEST. –提示Pass即可 FINGER DETECTION TEST(测试此项使用手指按住指纹感应器，测试合格显示FINGER DETECTION TEST=FPC_FINGER_PRESENT并自动跳入下一项；不合格显示FINGER DETECTION TEST=FPC_FINGER_ABSENT)点击相应项目后检查下方如果出现PASSED，则测试PASS；如果出现FAILED，则测试Fail.若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Finger Print Sensor, Select“SELF TEST, Verify test “PASSED”,Select “CHECKER BOARD TEST”,Verify test “PASSED”,Place finger on the Finger Print Sensor and then select “FINGER DETECTION TEST ”,Verify test result is “FPS_FINGER_PRESENT”,Press and Hold Volume Down key to pass and move to the next test;To fail the test, Press and Hold Volume Up key. - USB Type-C test reverse/ Type-C正反插测试/插入充电器，如果出现左图则PASS。然后拔出充电器，反向插入，测试结果如右图则PASS. 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 USB Type-C test.Insert Type-C usb, test result is shown as left picture, test pass. Reverse direction, test result is as shown as right picture, test PASS. Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key - Sequential Test Result:显示测试结果列表（绿色英文），检查测试结果，并记录不良品主机编号及其故障现象。按菜单键退回主屏。 28.Verify that all results show PASS. Failed tests will be in RED, Hit Back key to exit out of CQA App, Power off the phone, If CQA test FAILED, get phone analyzed If CQA test PASSED ,send phone to next station in the factory flow |
| 13 | 关机 Power off | 按关机键，屏幕显示Power off, 点击OK，关机，有振感；取出SIM卡和SD卡。 Press the Power Key, the screen shows the Power off, click OK, shutdown, a sense of vibration;Take out the SIM card. |

- Relevant documents/相关文件

| 序号 | 文件编号 | 文件名称 |
| --- | --- | --- |
| 1 | WHM-QA-Racer 5G-003 | Motorola Racer5G PSCD产品外观检验规范标准和品质保证 |
| 2 | WHM-QA-GN-101 | 手机和移动设备外观检验规范 |

- Relevant records/相关记录

| 序号 | 记录编号 | 文件名称 | 责任部门 | 保存方式 | 保存期限 |
| --- | --- | --- | --- | --- | --- |