JIMNY+ L10 CQA1 Inspection Working Instruction

JIMNY+ L10 CQA1检验作业指导书

| Files making and revising resume 文件制 / 修订履历表 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Maker/Reviser 制/修订人 | Making /revision content 制/修订内容 | Pages 影响页次 | Audit 审核 | Approve 批准 | Version 版本 | Issuing date 发行日期 |
| 陈泽伟 | 初版发行 | ALL | DCC | 姜静 | V1.0 | 2018/11/25 |

文件自发布之日起生效执行。

The file will come into effect since it is issued

文件使用者确保在使用前获取为最新版本。.

The file should be confirmed the latest edition before use

本文件最新版本保存在DPMS

The latest edition is kept in DPMS

打印版本仅供参考。

The paper edition is only for reference

对流程有修改意见的使用者，应通过邮件发送给文件负责人。.

The user having a suggestion on the process should send a email to the document owner

此文件为公司DPMS系统保密资产，请妥善保管

This document is the company's DPMS system confidential assets, please keep it

- 目的/Propose

Clarify Jimny+  CQA1 check items and methods in Lenovo Wuhan factory.

定义及规范武汉厂OSLO H  CQA1检验项目及方法,作为CQA1检验人员的依据。

- 适用范围/Scope

Document for  Jimny+  CQA1 inspectors in Lenovo Wuhan factory.

适用于武汉厂 Jimny+  CQA1人员

- 权责/Roles & Responsibilities

PQE is responsible for updating and releasing this document.

PQE负责本标准在武汉厂的更新和发布。

QC is responsible for failure confirmation and analyze and improvement

QC 负责对不良品的确认，分析及改善。

CQA is responsible for inspection follow WI.

CQA 负责按照作业指导书检验。

- 定义/Definitions

4.1 Class “AA” surface: Information display area only.

“AA” 级面: 只包括信息显示区域。

4.2 Class “A” surface: Primary surface exposed to direct close-up view in ordinary use.

“A” 级面: 暴露在外，正常使用中直接可见的主要表面。

4.3 Class “B” surface: Secondary surface exposed but not in direct view in ordinary use.

“B” 级面: 正常使用时，不能直接可见，暴露在外的表面。

4.4 Class “C” surface: Surface not visible in ordinary use.

“C” 级面: 正常使用不可见的表面

4.5 Class “D” surface: Non appearance surface, must meet protective and functional requirements.

“D” 级面:不可见的表面，必须满足防护和功能的要求。

Remark: Details surface classification please follow WHM-QA-GN-101Mobile phone and devices cosmetic inspection specification

备注：具体细节的级面定义以WHM-QA-GN-101手机和移动设备外观检验规范及PSCD为准。

- 作业内容/ Content

| Cosmetic and Accessory |  |  |
| --- | --- | --- |
| No | 检验项 Check items | 标准 Standards |
| 1 | Lens/LCD | 1.Review no scratch, no bubble and no indelible stains on Lens/LCD 检查Lens/LCD上是否有划痕、气泡和不可移除的污迹。 2.No hiatus on Lens/LCD and its model no. should match the phones’. 检查LENS无缺失，型号要与手机匹配。 3.The icon and letter should be clear and completely formed. No scratch，no indelible stains, no foreign material and tilt etc. LENS上的图标、文字要清晰、完整，要无划痕、不可移除的污迹、异物、翘起 Notice that there is no tilt around Lens ，ripple on Lens/LCD a 尤其注意检验Lens四周翘起，显示屏水纹现象。 |
| 2 | 防尘网 Mesh | EP/speaker has no hiatus, no dirty mark and no damages. 听筒/speaker防尘网无缺失，无污迹，无破损，凹陷等。 |
| 3 | 外壳 housing | 1.Front housing and rear housing should be better matched and be with the same color. 检查前后盖组装要匹配,颜色要匹配,安装要到位。 2.No damage, no scratch, no printing stains, no indelible dirty marks and no peeling off paint 检查外壳无损伤、划痕、印刷污迹、不可移除的污迹、无掉漆。 3. Notice that paint is not allowed on the edge of the battery cover and on the back trim 注意电池盖边沿及后摄装饰件不允许掉漆 |
| 4 | 开机键/ 侧键/卡槽 Power /side key/card slot | 1. Press side keys one by one, receive tactile response and be better assembled. 逐个按侧键，弹性无异常，组装到位。 2.No foreign materials, no damage, no indelible stains; be better assembled. 检查卡槽内无异物、无损伤、无不可移除的污迹，组装到位。 |
| 5 | 插口/插孔 Headset/USB jack | No foreign materials, no damage, no indelible stains etc. Focus on the headset empty inside the silicone sleeve is not exposed. USB插孔内无异物，无损伤，无不可移除的污迹。 |
| 6 | 摄像头 Camera | No skew, no damage, no indelible stains etc. 位置是否歪斜，不能有损伤，不可移除的污迹异物等。 |
| 7 | CIT 条码 CIT label | 1.Phone model no. must mach the label’s 核对手机型号与label是否一致。 2. Review the label and printing is clear and completely formed. 检查label是否丢失，印刷是否清楚、无断码。 |
| 8 | 单/双卡 Single/dual SIM | 1.Check the phone is with dual SIM or single SIM |
| 9 | 内存配置确认Storage confirm | Refer to XCVR comparing chart 参照料号对照表 |
| 10 | 其它未定义外观标准 Other Cosmetic Specification | WHM-QA-GN-101手机和移动设备外观检验规范 WHM-QA-GN-101 mobile phone and device cosmetic inspection specification |
| 11 | Function Check Items 功能检验项目 | Specification 标准 |
| 12 | 1. Read First 检前须知 | Please note the tests switch to next , after user input per 1 or 2 per below 检验前针对按上下音量键进入下一项测试，请注意以下几点： 1. VOL DOWN to pass the test 按下音量键：该测试项Pass 2. VOL UP to fail the test 按上音量键：该测试项Fail 3. There is a delay before the next test show after pressing either VOL up or down. DO NOT continue to press up or down key, this will continue to the next test and pass or fail it. You will need to wait. 在按上、下音量键进入下一项测试之前，会有短暂的停顿状态，不要长按，否则会进入下下测试，并且pass 或fail了上一项测试，必须等待。 4. In most of cases, you just need to press VOL up/down shortly to go to next test, in Key Test and headset Info test, you need to press a little bit long time to go to next one... because both of them need to test KEYs... Hold up or down key for 2 sec. for these tests when passing or failing them. 通常只需短按上、下音量键进入下一项测试。但在Key Test和Headset Info test中，必须长按不超过2秒钟进入下一项测试，因为短按只是在测试按键功能。 5. At the end of the Sequential test, a screen dump will show pass/fail tests 在Sequential Test测完后，屏幕上回显示绿色英文的测试结果列表。 |
| 13 | 2.插SIM卡和SD卡/安装电池/开机Insert SIM(s) and SD Card/ Install battery & turn on unit | 1. Insert SIM and SD/MMC Card. Please insert the SIM. 插入SIM卡和SD/MMC卡 2. Install battery and turn on unit with the power key with slight vibration. 按电源键开机,有轻微震感。 |
|  | 3.CQA 测试 CQA mode | Enter CQAmode: Select folder icon. Select Dialer icon. In dialer screen enter *#*#2486#*#*. Select Sequence Mode. And then select “YES”: 进入CQA测试模式，点击手机屏幕右下角“文件夹”图标，点击拨号图标，输入“*#*#2486#*#*”, 选Sequence Mode,弹出Confirmation对话框中选择“YES”开始测试： 插入耳机 - Headset Detect Status: PLUGGED - Headset Type: h2W - Has Microphone : YES - Headset Key Press Status: NA Headset/Headset Info: 显示 - Headset Detect Status: NA. - Headset Type: NA. - Headset Has Microphone: NA. - Headset Key Press Status: NA； 按耳机开关显示KEYCODE_ HEASSET HOOK pressed, 松开显示released； 拔下耳机显示如下:Headset/Headset Info: 显示 - Headset Detect Status: UNPLUGGED. - Headset Type: No Device. - Headset Has Microphone: No. - Headset Key Press Status: KEYCODE_ HEADSET HOOK -released 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。（注：需长按下音量键，才可进入下一个测试项，且最长只能按2秒钟） Press VOL DOWN key to pass TEST or VOL UP to fail TEST. (Remark: You may need to press the KEY longer than normal to get it to go to the next test. Please press for 2 seconds max.) - Key/Key Test：shows No Key is pressed. 侧键测试：显示No Key is pressed. 1)Hit Volume Up key: KEYCODE_VOLUME_ UP Pressed; release the key: KEYCODE_VOLUME _UP Released. 按上音量键显示KEYCODE_VOLUME_ UP Pressed, 松开显示 KEYCODE_VOLUME _UP Released。 2)Hit Volume Down key: KEYCODE_VOLUME DOWN Pressed; release the key: KEYCODE_VOLUME _UP Released. 按下音量键显示KEYCODE_VOLUME DOWN Pressed,松开显示 KEYCODE_VOLUME_ DOWN Released， 3)Press power key. Verify display goes dark and comes back with correct key code response: KEYCODE_POWER Pressed and Released。 按电源键，屏幕变暗后返回界面显示KEYCODE_POWER Pressed and Released。 4)Press VOL DOWN key to pass TEST or VOL UP to fail TEST. You may need to press a little longer to get to next test. 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - Bluetooth/ Bluetooth Scan：出现对话框，点击 Allow, 搜索蓝牙设备，若未收索到蓝牙设备，则此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Bluetooth/ Bluetooth Scan： Press the ALLOW button to turn on Bluetooth. Verify Bluetooth devices are discovered in the SCAN. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Bluetooth/ Bluetooth Mac：显示蓝牙地址，若未出现蓝牙地址，则此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Bluetooth/ Bluetooth Mac：Verify Bluetooth Mac address is displayed. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Ear speaker： - BUZZ Test：把手机放到耳边，能听到高低交错的长鸣音，检查是否有异常，如声音偏小，有杂音等。 - 再选择Play Harvard speed pattern，把手机放到耳边，能听到一段英文对话，检查是否有异常，如声音偏小，有杂音等。. - 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Ear speaker： - BUZZ Test: Listen for Buzz test audio from earpiece - Press Harvard Speech Pattern button；Listen for Harvard speech audio from ‘earpiece’ - Press VOL DOWN key to pass TEST or VOL UP to fail TEST. (‘Ear Speaker’) - Audio/Loudspeaker： 1) 点击Play Media File - testmidi.mid, 能听到喇叭音或音乐, 且声音清晰无杂音。 2) 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Audio/Loudspeaker： 1) Listen for media audio from ‘loudspeaker’; 2) Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Audio/Mic Loopback音频测试: - 插入耳机，点击DEFAULT MIC后, 选择第3项PRIMARY MIC, 把手机放在耳边，对着手机MIC说“喂喂喂 和 1234”，能从扬声器中听到清晰的回音，然后堵主手机下方mic1孔,测试漏音:（Mic 需使用耳机测试） OSLO H ROW漏音测试方法： 1、保证耳机插入OK； 2、切换到正确的MIC测试模式下进行测试； 3、保证手指完全将MIC孔堵住； 4、确保发音体到MIC孔10cm左右； 5、发音应当是正常打电话状态下的音量等级；（有些人说话声音超大，音频研发同 意此观点） OSLO H ROW漏音测试标准：堵孔前后有明显差异即可。 - 点击DEFAULT MIC后，选择第4项SECONDARY MIC, 把手机放在耳边，对着手机MIC说“喂喂喂 和 1234”，能听到清晰的回音。然后堵主手机下方mic1孔,测试漏音,方法与Mic1漏音测试方案一致（Mic 需使用耳机测试） - 点击DEFAULT MIC后，选择第4项Headset mic, 插入耳机，对着耳机MIC说“喂喂喂 和 1234”，能听到清晰的回音。 - 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Audio/Mic Loopback: - Press DEFAULT MIC and then press the third item: PRIMARY MIC; Put phone to ear and talk into Mic, listen for your voice from ‘earpiece’ Then plug the mic1 hole in the lower part of the main phone, and test the leakage :(Mic needs to use headset test) 1. Ensure that the headset is disassembled. 2. Switch to the correct MIC test mode for testing; 3. Ensure that the MIC hole is completely blocked by the finger; 4. Ensure that the articulator reaches about 10cm from the MIC pore; 5. The pronunciation should be the volume level under normal phone calls;(some people speak really loud, and that's the idea behind audio development.) - Press DEFAULT MIC and then press the fourth item: SECONDARY MIC; Put phone to ear and talk into Mic, listen for your voice from ‘earpiece’(Use headset to test Mic2) - Press DEFAULT MIC and then press the third item: Headset mic; Plug the headset into the headset hole and talk into headset microphone, listen for your voice from ‘earpiece’ - Press VOL DOWN key to pass TEST or VOL UP to fail TEST(‘audio/MIC Loopback’) 8.SIM Card电话卡测试: 1）检查SIM serial number：SIM卡序列号出现即可，如下图。 2）若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 SIM Card: 1) Verify SIM serial number is displayed. Please note it may say UNKNOWN for the state depending on SIM used, and that is a PASS. 2) Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - MMC/SD Card self Test： 1) 选择Query MMC current state，检查MMC card 状态为Mounted。 2) 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 MMC/SD Card self Test： 1) Select ‘Query MMC current state’; Verify ‘MMC Card: Mounted’ 2) Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Version软件版本测试： 1）检查AP Version，BP Version， CQA Test Package Version是否完整 2）信息不全则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Version： 1）Verify that the AP, BP and CQA version 2）Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Display显示测试：（主要检查显示屏同色点，异色点，划伤，黑底白框断线，缺划，透光，若有其它缺陷，则视为FAIL，除非有限度样本。） 1）低亮度六色块-->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 2）高亮度六色块-->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 3）灰白界面->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 4）黑色界面->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 5）白色界面->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 6）黑底白框界面->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Display: Check same color dot, contrasting color dot, scratch. Other defects are rejected unless limit samples are approved by Motorola: - Review ‘Medium Color Bar’ for defects -> Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Review ‘Bright Color Bar’ for defects -> Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Review ‘Greyscale ’ for defects -> Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Review ‘Black screen’ for defects -> Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Review ‘White screen’ for defects -> Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Review ‘White with black border’ for defects -> Press VOL DOWN key to pass TEST or VOL UP to fail TEST. 暗六色块 亮六色块 灰色 Meduim color bar Bright color bar Grayscale 黑色 白色 黑底白框 Black White White with black border - Touch Screen触屏测试：用手指点击白色圆点，沿着白色路径划线，划完后，检查屏幕是否变绿，变绿后将自动进入下一项测试。 Touch Screen: Trace the path with a finger following the white dot until screen turns green indicating pass; Verify screen turned green and it will thereafter sequence automatically to the next test - Sensor/ Front Light光感测试: 显示绿色英文：Light Sensor：Normal 1) 用手或手指遮挡Light sensor（位于手机顶部）, 显示Light Sensor: Dark’. 2) 手或手指移开light sensor后，显示‘Light Sensor: Bright/Brigthter/Brightest。 3）若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Sensor/ Front Light: shows Light Sensor：Normal - Verify that blocking the light sensor (located at the top of the phone) with a hand or finger causes phone to indicate ‘Light Sensor: Dark’. - Verify that if light sensor is not blocked phone indicates ‘ Bright/Brigthter/Brightest’. - Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Sensor/proximity距离感应器测试： 1) 用手遮到感应点，显示绿色英文“Proximity Sensor: Enable”， 2) 拿开手, 显示红色英文“Proximity Sensor: Disable”后自动进入下一项测试,若不出现Enable或Disable或出现其他异常，此次测试FAIL, 则按上音量键表示Fail； Sensor/proximity： - Verify that blocking the proximity sensor (located at the top of the phone) with a hand or finger causes phone to indicate ‘Proximity Sensor: Enable’. - Verify that if proximity sensor is not blocked phone indicates ‘Proximity Sensor: Disable’ and it will thereafter sequence automatically to the next test. Press VOL UP to fail TEST. - Sensor/Gyroscope 陀螺仪感应测试 移动手机，白色数值变化即可。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Sensor/Accelerometer：Verify that movement of the phone (turning and twisting its orientation) changes the x, y, and z numbers. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Sensor/Accelerometer加速度传感器测试： 移动手机，白色数值变化即可。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Sensor/Accelerometer：Verify that movement of the phone (turning and twisting its orientation) changes the x, y, and z numbers. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - WLAN/Scan Networks: 显示网络名称。若未收索到，则此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 WLAN/Scan Networks: Verify Networks are discovered and Network list appears. Press VOL DOWN key to pass TEST or VOL UP to fail TEST - WLAN:/WiFi Mac：显示网络地址即可。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 WLAN:/WiFi Mac：Verify WiFi Mac Address is displayed. Press VOL DOWN key to pass TEST or VOL UP to fail TEST - Vibratory马达测试: 按“OFF”有震动，按“ON”震动停止，检查震动是否停止，不能出现震动异音，震动异常等。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Vibratory: Touch On / Off switch to turn vibrator on and verify vibrator is on; Touch On / Off switch to turn vibrator off, and verify vibrator is off. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Battery/Battery Info: 插上充电器，检查电池图标里是否有闪电图标，并检查前6项符合如下标准（粗体）。 - Battery Voltage >3.000V, - Battery Healthy: BATTERY_HEALTHY_GOOD. - Battery Plugged: BATTERY_ PLUGGED_AC - Battery Present: TRUE 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试,检查完拔下充电器 1.电量＜50%时，屏幕背景色为黑色。 2.50% ≤电量 <60%时，屏幕背景色为红色。 3. 60% ≤电量 <80%时，屏幕背景色为黄色。 4.电量≥80%时，屏幕背景色为绿色。 Press VOL DOWN key to pass TEST or VOL UP to fail TEST. Then unplug charger. - Battery Level＜50%, the color definition is black. 2. 50%≤Battery Level <60%, the color definition is red. 3. 60% ≤Battery Level <80%, the color definition is yellow. 4. Battery Level≥80% , the color definition is green. 23.USB Host Mode test： OTG 测试：手机显示红色字体“Device is NOT in USB HOST MODE”时，将外设OTG插入USB口，显示屏测试画面为绿色字体“Device is in USB HOST MODE”则表示通过，反之不通过；测试pass后按音量下键进入下一个测试项 USB HOST MODE test：Mobile phone display red font "Device is NOT in USB HOST MODE", insert the peripherals OTG USB port, screen test picture for the green font "Device is in USB HOST MODE", said by, whereas NOT through; According to the volume down button to enter after test pass the next test items - Camera多媒体测试： - 点击 “start capture” 开始录像，对着手机说‘喂喂喂和1234’，最多录10秒后，点击“stop capture”停止录像。点击“start playback”播放录像，查看录像是否清晰，声音是否正常，点击“stop playback”停止播放。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - 进入后置摄像头拍照状态，点击屏幕任意一处进行拍照，注意听是否有“咔嚓”一声响（注：若没听到“咔擦”声，请注意音量是否调至最大）且闪光灯闪亮。拍照时，检查是否有异常，如模糊，异色，缺划等，若不能拍照，无咔嚓声或闪光灯不亮或存在其他缺陷，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - 进入后拍闪光灯测试，注意闪光灯正常亮起，若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - 进入后拍预览测试，能正常预览即可，若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - 进入前摄像头录像状态，点击 “start capture” 开始录像，对着手机说‘喂喂喂和1234’，最多录10秒后，点击“stop capture”停止录像。点击“start playback”播放录像，查看录像是否清晰，声音是否正常，点击“stop playback”停止播放。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - 进入前置摄像头拍照状态，点击屏幕任意一处进行拍照，注意听是否有“咔嚓”一声响（注：若没听到“咔擦”声，请注意音量是否调至最大），拍照时，检查是否有异常，如模糊，异色，缺划等，若不能拍照，无咔嚓声则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - 进入前置摄像头预览状态，检查取景是否有异常，如模糊，异色，缺划等，若有异常，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Camera test: 1) click "start capture" to start the video. Say "feed and 1234" to the phone. After 10 seconds, click "stop capture" to stop the video.Click "start playback" to see if the video is clear, if the sound is normal, and click "stop playback" to stop playback.If FAIL in this test, press the volume button to indicate FAIL.If PASS, press the volume button to enter the next test. 2) enter the post camera to take photos, click anywhere on the screen to take photos, and pay attention to whether there is a "click" sound (note: if no "click" sound is heard, please pay attention to whether the volume is turned to the maximum) and the flash flash.When taking the picture, check whether there is any abnormality, such as blurring, different colors, missing scratches, etc. If the picture cannot be taken, no click sound or flash flash or other defects exist, then FAIL this test. Press the volume button to indicate FAIL.If PASS, press the volume button to enter the next test. 3) click the flash light test after entering, and notice that the flash light is normally on. If FAIL in this test, press the volume button to indicate FAIL.If PASS, press the volume button to enter the next test. 4) press the volume button to indicate FAIL.If PASS, press the volume button to enter the next test. 5) enter the video state of the front camera. Click "start capture" to start the video. Say "feed and 1234" to the phone.Click "start playback" to see if the video is clear, if the sound is normal, and click "stop playback" to stop playback.If FAIL in this test, press the volume button to indicate FAIL.If PASS, press the volume button to enter the next test. 6) into a front-facing camera taking pictures, click on the screen any place to take photos, listen to see if there is the sound "click" (note: if you didn't hear "cut brush" sound, please pay attention to whether the volume to maximum), take pictures, to check whether there are abnormal, such as fuzzy, different color, such as lack of stroke, if no photos, no clicks, the test FAIL, press on the volume FAIL;If PASS, press the volume button to enter the next test. 7) enter the preview state of the front-facing camera to check if there is any abnormality in the scene selection, such as blurring, different colors, missing scratches, etc. If there is any abnormality, FAIL this test. Press the volume button to indicate FAIL.If PASS, press the volume button to enter the next test. 25.Sensor/Fingerprint Test: 1.进入指纹测试界面后等待系统提示“Please ,PLACE your finger on the sensor and wait for result ”，将手指放在指纹模组上测试，测试pass 后系统自动跳转下一步 1. After entering the fingerprint test interface, wait for the system to prompt "Please,PLACE your finger on the sensor and wait for result", put your finger on the fingerprint module for testing, and the system will jump to the next step after testing the pass 26. Capsense test /电容感应测试 感应测试：如手机显示如下左图红色方框内容时，使用大拇指、食指拿取手机中间，避开手机TOP(顶)、Bottom（底部），当测试内容显示为右图时使用手接触手机顶部或底部（箭头指向位置内容Top 接触顶部，显示 Bottom接触底部）当测试项自动跳过，进入下一项测试，则为PASS，不进入下一项，或报错，则为Fail.按音量上键判定为Fail. Sensor/CapsenseMul：Verify that movement of the phone (turning and twisting its orientation) changes the x, y, and z numbers. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. 27. Type-C reverse test/Type-C 翻转测试 使用type-C 数据线正反插拔USB各一次，如 测试结果为”YES”,则测试结果为pass,如返回值为”NO”，则测试结果fail Use type-C cable to plug USB positive and negative each time. If return value ”YES”,test result is pass ; if return value ”NO”,test result is fail - Sequential Test Result: 显示测试结果列表（绿色英文），检查测试结果，并记录不良品主机编号及其故障现象。按菜单键退回主屏。 Sequential Test Result: The result LIST will now appear on the phone screen. Check and record ID no. and Defect Symptom of rejected units. Press home key to return to menu. |
| 14 | 关机 Power off | 长按电源键关机，屏幕完全熄灭后取出SIM卡/SD/MMC卡/(禁止直接拔USB关机) Unplug the charging cable off. After the screen is completely out, remove the SIM 、SD/MMC card |

- Relevant documents/相关文件

| 序号 | 记录编号 | 文件名称 | 责任部门 | 保存方式 | 保存期限 |
| --- | --- | --- | --- | --- | --- |
| 1 | WHM-QA-GN-101 | 手机和移动设备外观检验规范 | QA | 纸档 | 1年 |
| 2 |  | JIMNY + PSCD 产品外观检验规范标准和品质保证 | QA | 纸档 | 1年 |

7  Relevant records/相关记录

NA 无