MOTO One Vision

Trouble Shooting Guide

V1.0

Contents

1. MB/KB illustration	2

2. Power on failure	4

3. Screen is not bright	6

4. Screen display abnormality	7

5. Touch screen failure	9

6. Optical/distance sensor error	10

7. Gravity sensing failure	11

8．Key failure	12

9. No charging	13

10. Main camera function failure	16

11. Slave camera function failure	17

12. The previous camera function failed	18

13. Flashlight failure	18

14. Motor error	19

15. GSM network failure	20

16. WCDMA& LTE network failure	21

17. WIFI/ Bluetooth /GPS	22

18. Radio failure	24

19. Headset identification failure/headset MIC errors	25

20. MIC silent/low volume	26

21. Earpiece silent	26

22. Speaker silent	27

23. Phone Can't Access SIM Card	28

24. Phone Can't Access T-flash Card	29

# 1. MB/KB illustration

Rear main camera interface

Rear auxiliary camera interface

Audio jack

FPS interface

Screen connector

MB-KB FPC connector

Backlight IC

Bias voltage IC

MOTO IC

NFC LDO

NFC

JTAG debugging interface

Four in one chip

battery connector

Sar sensor

MB MIC

TX/RX FEM

Transivor

switch（HB）

Coaxial coordinates

WIFI FEM

26M TCXO

5G FEM

Antenna switch

Mid - low frequency diversity

High frequency diversity

Coaxial coordinates

3G/4G PA

APT IC

Proactive interface

SIM/SD

SmartPA

Charger

OVP

Main Charger

PMU

A+G sensor

The side key seat

Geomagnetism sensor

Codec IC

UFS

CPU+DDR_INPMIC

SPK shrapnel

USB

Link-fpc connent

MIC

The motor shell

# 2. Power on failure

Solution:：

- Long press the switch key for 30s to restart the machine；

- Insert the charger and charge it for 5 minutes. Is there any display on the screen and check whether the battery VBAT is short of power (less than 3.5v)；

- Check battery connector welding and body for oxidation or damage；

- Check whether the VBAT voltage below is short circuit or abnormal voltage with GND (VBAT line TVS tube, voltage regulator tube and so on are breakdown to the ground)；

- Check whether switch key connector and switch key FPC are damaged；

- Test the startup time sequence for any other signal abnormalities：

KYPD_PWR_N1--------- Switch signal from 0V to 4V

PON_RESET_N--------- Starting reset signal 0V jump to 1.8v

PS_HOLD ---------------- Power maintenance control signal 0V jump to 1.8v

- Check the voltage in table 2.1 below for GND short circuit or abnormal voltage：

Table 2.1 Power-on sequence: SDM636

| Power-on-sequence | Device location | Regulator name | Vout primary (V) | Notes |
| --- | --- | --- | --- | --- |
| 1 | C1564 | VLDO1_PMIC_0P75 | 0.75 V |  |
| 2 | C1571 | VLDO2_PMIC_2P8 | 2.8 V |  |
| 3 | C1574 | VLDO3_PMIC_1P8 | 1.8 V |  |
| 4 |  | VLDO4_PMIC_0P6 | 0.6 V |  |
| 5 | C1579 | VLDO5_PMIC_0P85 | 0.85V |  |
| 6 | C1501 | VLDO6_PMIC_1P2 | 1.2 V |  |
| 7 | C1502 | VLDO7_PMIC_1P8 | 1.8 V |  |
| 8 | C1592 | VLDO8_PMIC_0P75 | 0.75 V |  |
| 9 | C1504 | VLDO9_PMIC_0P75 | 0.75 V |  |
| 10 | C1505 | VLDO10_PMIC_0P75 | 0.75 V |  |
| 11 | C1506 | VLDO11_PMIC_0P75 | 0.75 V |  |
| 12 | C1507 | VLDO12_PMIC_0P85 | 0.85 V |  |
| 13 | C1508 | VLDO13_PMIC_1P8 | 1.8 V |  |
| 14 | C1509 | VLDO14_PMIC_3P0 | 3.0 V |  |
| 15 | C1510 | VLDO15_PMIC_0P8 | 0.8 V |  |
| 16 | C1511 | VLDO16_PMIC_1P8 | 1.8 V |  |
| 17 | C1512 | VLDO17_PMIC_1P2 | 1.2V |  |
| 18 | C1513 | VLDO18_PMIC_UIM0 | 1.8/2.85V |  |
| 19 | C1514 | VLDO19_PMIC_UIM1 | 1.8/2.85V |  |
| 20 | C1515 | VLDO20_PMIC_1P8 | 1.8V |  |
| 21 | C1517 | VLDO22_PMIC_1P2 | 1.2V |  |
| 22 | C1518 | VLDO23_PMIC_1P2 | 1.2V |  |
| 23 | C1519 | VLDO24_PMIC_1P8 | 1.8V |  |
| 24 | C1525 | VLDO25_PMIC_2P8 | 2.8V |  |
| 25 | C1526 | VLDO26_PMIC_1P1 | 1.1V |  |
| 26 | C1520 | VLDO27_PMIC_1P1 | 1.1V |  |
| 27 | C1527 | VLDO28_PMIC_1P2 | 1.2V |  |
| 28 | C1528 | VLDO29_PMIC_1P8 | 1.8V |  |
| 29 | C1521 | VLDO30_PMIC_1P8 | 1.8V |  |
| 30 | C1529 | VLDO31_PMIC_3P3 | 3.3 V |  |
| 31 | C1530 | VLDO32_PMIC_3P3 | 3.3 V |  |
| 32 | C1522 | VLDO33_PMIC_1P8 | 1.8V |  |
| 33 | C1531 | VLDO34_PMIC_2P95 | 2.95 V |  |
| 34 | C1533 | VLDO35_PMIC_SDCARD | 3.0 V |  |
| 35 | C1535 | VLDO36_PMIC_0P75 | 0.75 V |  |
| 36 | C1523 | VLDO37_PMIC_FPS_3P3 | 3.3 V |  |
| 37 | C1524 | VLDO38_PMIC_RCAM_AFVCC_2P8 | 2.8 V |  |
| 38 | C1536 | VLDO39_PMIC_LCD_1P8 | 1.8 V |  |
| 39 | C1537 | VLDO40_PMIC_SENSOR_3P0 | 3.0V |  |
| 40 | C1538 | VLDO41_PMIC_FCAM_AVDD_2P8 | 2.8V |  |
| 41 | C1539 | VLDO42_PMIC_CODEC_1P8 | 1.8V |  |
| 42 | C1540 | VLDO43_PMIC_0P75 | 0.75V |  |
| 43 | C1541 | VLDO44_PMIC_DCAM_DVDD_1P2 | 1.2 V |  |

- If the starting current is about 20MA, check whether the 26M crystal oscillator circuit is welded normally；

- If the startup current is abnormal above 1A, check whether the VBAT circuit is connected to the ground, and investigate the grounding diodes at various positions of the VBAT circuit；

- If the startup current is abnormal at about 100MA, please touch the main board to check whether there is any heating position；

- X-ray check the welding condition of each main chip,CPU/Flash/PMIC；

- Add welding or reweld the main chip to confirm the welding status；

- Replace CPU/Flash/PMIC;

# 3. Screen is not bright

Solution：

- Press the "on/off" button to check whether there is motor vibration to start up or plug in the charger；

- Check whether LCM connector assembly is normal and reassemble；

- Adjust the normal LCM display screen and eliminate the LCM single problem point；

- Check whether R2661 and R2662 are on normally, and the voltage output is +/ -5.5v (the display drives the boost circuit);

- Check whether there is 24v-26.4v voltage in B2603 of backlight boost circuit, and replace L2620 and D2611 successively；

# 4. Screen display abnormality

Solution：

- Reassemble the LCD display or replace the LCD and restart the machine for observation;

- Check whether the welding or body of LCD connector is abnormal;

- Inspect the common mode filter and the LCD circuit resistance (FL2608 FL2609, FL2610, FL2611, FL2612) whether there is abnormal phenomenon such as welding or damaged;

- Check whether the CPU has blown rubber, and X-ray equipment can be used to observe the situation of IC tin ball;

- Add welding/re-welding CPU or replace;

LCM Common mode filtering position

# 5. Touch screen failure

Solution：

- Reassemble TP or restart the test after replacement;

- Check whether there is any abnormity in the welding or body of LCD connector J2601, and test after re-welding or replacement;

- X-ray inspection of the CPU welding, welding or re-welding after testing;

- Multimeter is used to measure the ground resistance of TP test signal point compared with the normal plate;

- Replace the CPU;

# 6. Optical/distance sensor error

Solution：

- Check whether the surface of the light sensor chip is stained or blocked;

- Inspect the light sensor/distance sensor body for damage or abnormal welding;

- Measure whether the voltage of VLDO3_PMIC_1P8 is normal;

- Measure whether the voltage of VLDO40_PMIC_SENSOR_3P0 is normal;

- Check whether there is pipe pressure drop in each shrapnel (path) of optical distance sensing;

- Replace the optical distance FPC on the A shell

- Add welding CPU/PMIC to re-weld, or replace;

Light + distance sensor shrapnel

# 7. Gravity sensing failure

Solution：

- Measure whether the voltage on VLDO3_PMIC_1P8 is 1.8v

- Resolder or replace the gravity sensor U2503;

- Check whether the circuit elements of gravity sensing module have bad welding or body damage

- Resolder or replace the CPU;

Acceleration + gyroscope two in one sensor

# 8．Key failure

Solution：

- Test after reassembling the connector or replace the side key;

- Check whether the voltage signal of power key KYPD_PWR_N1, volume +KEY_VOL_UP and volume -KEY_VOL_DOWN is normal;

- Check whether the components on the key circuit are damaged or short-circuited to the ground or poorly welded;

- Re-weld or replace the CPU and PMIC；

Key connector

# 9. No charging

Solution：

- Reconnect the KB board or replace the main and auxiliary board with FPC for testing;

- Check whether the board connector on the main board is stained or damaged;

- Test after battery replacement;

- Check whether battery connector J3201 is dirty, spurious or damaged;

- Check whether the charging temperature rise protection chip NMOS is abnormal and exchange the normal plate for verification;

- Check whether the OVP overvoltage protection chip U3202 and U3203 are abnormal and exchange the normal board for verification;

- Check whether the TVS of USB_VBUS_IN, USB_IN_CON, USB_OVP_IN line short-circuits to the ground, etc.;

- Check whether the common mode filter of the USB circuit of the small board is damaged;

- Add welding u1201-s2mu106 power management chip or replace;

- Welding u1202-s2mcs02 charging chip or replacement;

- Resolder or replace the CPU;

Main Charger

PMU

charger

# 10. Main camera function failure

Solution：

- Check whether the Camera is abnormally assembled, and reassemble or replace the Camera for retest (when the problem of single main rear Camera is excluded, pay attention to removing the front Camera and rear radiography when replacing the Camera -- the common iovdd1.8v with the main rear Camera is also removed).

- Check whether the connector of main camera J2712 and common mode filter resistance of peripheral devices (U2713/ U2714/ U2717/ U2718/ U2719) are damaged or abnormal in welding, and re-weld or replace the connector;

- Check the voltage of REAR_CAM_AVDD_2P8/ CAM_REAR_DVDD_1P05/ CAM_IOVDD_1P8;

- Check whether the LDO chips U2704,U2731 and U2705 are damaged or abnormal in welding;

- Add welding/re-welding PMIC, or replace;

- Add welding/re-welding CPU, or replace;

# 11. Slave camera function failure

Solution：

- Check whether the Camera is abnormally assembled, reassemble or replace the Camera for retest (when the single rear Camera head is checked, the front Camera and the main Camera are removed when the Camera is replaced -- common iovdd1.8v with the secondary rear Camera head);

- Check whether the connector of secondary camera J2702 is damaged or abnormal in welding, and re-weld or replace the connector;

- Check whether the voltage of CAM_FRT_AVDD_2P8 /VLDO44_PMIC_DCAM_DVDD_1P2/CAM_IOVDD_1P8 is abnormal. If the voltage is abnormal, check whether the PMIC output is abnormal or whether there is a short circuit or open circuit in the peripheral circuit.

- Check LDO chip U2701,U2729 welding or ontology is normal;

- Add welding/re-welding CPU, or replace;

Secondary rear camera and common mode filtering

Main rear camera and common mode filter

# 12. The previous camera function failed

Solution：

- Check whether the Camera is abnormally assembled, and reassemble or replace the Camera for retest (remove the main rear Camera and rear radiography when replacing the Camera);

- Check whether the connector of auxiliary camera J2701 is damaged or abnormal in welding, and re-weld or replace the connector;

- Check whether the voltage of CAM_FRT_DVDD_1P05 /CAM_IOVDD_1P8/CAM_FRT_AVDD_2P8 is abnormal, and if so, check whether the PMIC output is abnormal or whether there is a short circuit or open circuit in the peripheral circuit.

- Check whether the LDO chip U2701 and U2729 welding or the body are normal;

- Add welding/re-welding CPU or replace;

# 13. Flashlight failure

Solution：

- Add about 3.0v voltage at both ends of the LED pad to test whether the LED lamp can be lit;

- Replace the flash plate, or check whether the structural shrapnel in contact with the flash plate is out of position and damaged the lamp;

- Resolder or replace PMIC;

# 14. Motor error

Solution：

- Measured the voltage values of ANT201 and ANT202 shrapnel on KB board, and confirmed them by comparing with the normal motherboard;

- Check whether the devices of positive and negative paths of the motor are damaged, short-circuit to the ground, etc.;

- Resolder the motor or test after replacement;

- Resolder motor chip U1203 or replace it;

# 15. GSM network failure

Solution：

- Check whether the antenna on the KB plate is welded, in contact with the antenna of the mobile phone shell or damaged;

- Check whether there are abnormal welding phenomena of inductive resistance elements of RF connector connecting circuit after parts collision;

- Check whether the welding of front-end rf switch FEM3301 and related components is abnormal or the body shows signs of damage;

- Re-calibrate the rf index of the main board, and troubleshoot and maintain the failed frequency band if failure occurs;

- Add welding or replace U4001 rf transceiver chip;

- Add welding or replace FEM3301 rf devices;

# 16. WCDMA& LTE network failure

TX/RX FEM

Transivor

Solution：

- Check whether the antenna on the KB plate is welded, contacts with the antenna of the mobile phone shell or the body is damaged, and whether the coaxial line is broken;

- All frequency band transmission and main set reception have problems, add welding or replace main components of the channel;

- Solder or replace switch SW3701 if there is a problem with diversity reception of all bands

- Band1/2/3/4/5/7/8 12/17/28/66 launch reception has a problem, plus welding or replace the filter, FEM3301, U4001;

- If it is the problem of a band, the mobile phone motherboard shall be recalculated to find the source of the error. According to the frequency band of the corresponding problem, it is judged that the problem is the failure of reception or transmission, and the corresponding loop element shall be investigated.

- Sorting out the problem of a certain frequency band according to the schematic diagram;

CPU

# WIFI/ Bluetooth /GPS

Solution：

- Whether the antenna is damaged due to welding, contact with the antenna of the mobile phone shell or the body;

- Test block J4301, J4308 whether abnormal;

- BT/WIFI FAIL checks whether there are obvious welding abnormalities in the devices on the corresponding path, checks whether there are welding problems in U4301, FL4303, SW4301 and WIFI main chip, and changes or adds welding to the main devices of the path.

- GPS FAIL check access, resolder or replace U4301, FL4301, FL4302, LNA4301 and other key devices;

- 5G WIFI fail check access, resoldering or replacement of SW4301, FL4308, FEM4302, SW4301 and other key devices;

# 18. Radio failure

Solution：

- Retest FM function after replacing American standard headphones;

- Check whether the earphone holder has welding deviation, floating height or body damage and other adverse phenomena;

- Check whether the earphone ground components of C2350, R2310 and B2305 are damaged, collision parts, virtual welding and short circuit to the ground;

- Check whether the FM_OUT_P headphone antenna channel device is damaged, damaged or not, and if it is not soldered;

- Check whether the earphone's left and right channel channel devices are damaged, collision parts, virtual welding and short circuit to the ground;

- Add welding/re-welding U4501 WLAN/BT/FM three-in-one chip, or test after replacement;

- Add welding/re-welding CPU, or test after replacement;

# 19. Headset identification failure/headset MIC errors

Solution：

- Retest the headset function after replacing the headset;

- Check whether the headset holder has any bad phenomena such as welding deviation, floating height or body damage, and resolder or replace the headset holder;

- Check whether the earphone ground loop components have welding defects or body damage, and test after repair;

- Check whether the earphone interrupt signal CDC_HPH_DET circuit device has welding defects or body damage, and then test after repair;

- Check whether the MIC signal HPH_MIC_P circuit device of the headset has poor welding or body damage, and then test after repair;

- Check whether there is any welding defect or body damage in the circuit device of the left and right sound channel of the headset, and test after repair;

- Resolder or replace CPU

# 20. MIC silent/low volume

Solution：

- Main MIC is on the KB board, replace the KB board or the positioning problem point of the main board or the small board;

- Check whether the main MIC negative grounding resistance is falsely welded and damaged;

- Check whether the connector of MB KB board has bad phenomena such as welding deviation or tin or body damage;

- Check whether there is any welding defect or body damage of MIC related small components;

- Test was conducted after the secondary MIC was replaced. When the secondary MIC was replaced, the MIC body was damaged by high temperature.

- Add welding/re-welding PMIC, or test after replacement;

# 21. Earpiece silent

Solution：

- Reassemble or replace the receiver for testing;

- Visually check whether the earpiece circuit passage element has any bad welding or body damage, etc.;

- Add welding or replace PMIC

# 22. Speaker silent

Solution：

- Horn on the KB board, replace the KB board for testing to determine the problem points on the KB board or the main board after maintenance;

- Visually check whether the audio PA U2306 and peripheral devices have poor welding or body damage;

- Check audio output and input channel devices for welding defects or body damage;

- Check whether the connector between the main board and KB board has any welding deviation, tin and other bad phenomena;

- Add welding/replace Smart PA U2306;

- Add welding/replace Codec U1802;

- Add welding or replace PMIC U1501/CPU U0401.

Smart PA

# 23. Phone Can't Access SIM Card

Solution：

- Reassemble the SIM card or test after replacement;

- Check whether the SIM card holder has bad phenomena such as welding deviation, floating height or body damage;

- Check whether SD_SIM_DET (R2912.2), the hot plug interrupt signal of SIM card and T card, is abnormal (if there is no plug card, the signal is low; if there is plug card, the signal is high)

- Measure SIM power supply voltage VLDO18_PMIC_UIM0 and VLDO18_PMIC_UIM0;

- Resolder or replace PMIC;

- Resolder or replace the CPU;

# 24. Phone Can't Access T-flash Card

Solution：

- Reassemble T card or test after replacement;

- Check whether the T holder is defective such as welding deviation, floating height or body damage;

- Check whether SD_SIM_DET (R2912.2), the hot-plug signal of the T card holder, is abnormal (the signal is low if there is no card holder, and high if there is card holder).

- Measure the supply voltage of T, VLDO35_PMIC_SDCARD;

- Resolder or replace PMIC;

- Resolder or replace the CPU;