WHP QUATTRO BE CQA1 检验作业指导书

| Files making and revising resume 文件制/修订履历表 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Maker/Reviser 制/修订人 | Making and revision content 制/修订内容 | Pages 影响页次 | Audit 审核 | Approve 批准 | Version 版本 | Issuing date 发行日期 |
| 柯聪 | WHP Quattro BE CQA检验作业指导书 | ALL |  |  | V1.0 |  |

文件自发布之日起生效执行

文件使用者确保在使用前获取为最新版本

本文件最新版本保存在DPMS

打印版本仅供参考

对流程有修改意见的使用者，应通过邮件发送给文件负责人

此文件为公司DPMS系统保密资产，请妥善保管

- Propose/目的

定义及规范联想武汉厂QuattroCQA检验项目及方法,作为CQA检验人员的依据。

ClarifyQuattroCQA checking items and contents in Lenovo Wuhan factory.

2.Scope/适用范围

适用于武汉厂QuattroQC人员

Document forQuattroQC inspectors in Lenovo Wuhan factory

3.Roles & Responsibilities/权责

PQE负责本标准在武汉厂的更新和发布

PQE is responsible for updating and releasing this document.

4.Definitions/定义：

| 表面等级 分类 Surface classification | - “AA” 级面: 只包括信息显示区域以及前后摄像头成像区域和以及前后摄像头非成像区域 Class “AA” surface: Information display area only and Camera View Area& Non View Area. - “A” 级面: 暴露在外，正常使用中直接可见的主要表面(包括摄像头塑料区域)。 Class “A” surface: Primary surface exposed to direct close-up view in ordinary use and Front and rear cameras can't imaging - “B” 级面: 正常使用时，不能直接可见，暴露在外的表面。 Class “B” surface: Secondary surface exposed but not in direct view in ordinary use. - “C” 级面: 正常使用不可见的表面 Class “C” surface: Surface not visible in ordinary use. - “D” 级面:不可见的表面，必须满足防护和功能的要求。 Class “D” surface: Non appearance surface, must meet protective and functional requirements. 6.详细定义见PSCD Detail Definition as PSCD: |
| --- | --- |

- Content/作业内容

| Cosmetic and Accessory 外观和辅料 |  |  |
| --- | --- | --- |
| No | 外观检验 Cosmetic inspection | 标准 Standard |
| 1 | 屏幕/后盖 Display/Inlay | - 检查是否有划痕、气泡和不可移除的污迹。检查屏幕有无缺失，型号要与手机匹配。安装到位，无缺损、刮花、不可移除的污迹、掉漆、间隙，段差等不良。边缘无溢胶、漏铜等（除非有样本）。 - 屏幕上的图标、文字要清晰、完整，无划痕、不可移除的污迹、异物、翘起 - 尤其注意检验Lens、Plastic四周翘起、松动、边框胶爬墙、溢胶以及显示屏水纹现象。 - 注意Lens MIC处是否偏移、异物、堵孔、露白等不良现象。 |
| 2 | 防尘网 | 听筒/扬声器防尘网无缺失，无污迹，无损伤，无下陷等不良现象。 |
| 3 | 后壳 | - 检查前后盖组装要匹配，颜色要匹配，安装要到位。 - 检查外壳无损伤、划痕、印刷污迹、不可移除的污迹、无掉漆，无翘起等不良。 |
| 4 | 开机键/音量键/卡托 | - 检查按键颜色与料号要匹配。逐个按侧键，弹性无异常，无下陷，组装到位。 - 检查卡托（装卡前，装卡后）无凸起，无下陷，拨杆无断裂，无掉漆等不良现象。 - 检查SIM单双卡信息以及颜色与主机头对应正确。 |
| 5 | 插口 | 孔内无异物，无损伤，无不可移除的污迹；插拔无阻塞。 |
| 6 | 摄像头/闪光灯/IR/LED | 1.检查摄像头是否歪斜，不能有损伤，不可移除的污迹异物等。 2. 检查闪光灯和LED灯无缺损，无不可移除的污迹异物，亮度无偏暗等。 3. 检查IR孔内无不可移除的脏污异物等。 |
| 7 | 标签 | 1. 检查label是否丢失，印刷是否清楚、无断码。 2. CQA模式下的barcode与Label Tracking ID一致，主机物料编号与MES一致。 |
| 8 | 其它外观标准 | 参照《MBG-S-PQM-061外观检验标准》、《SMT外观检验标准》、《Quattro 6.39FHD_Cosmetic Inspection Condition and Criteria_for Lenovo_v1.0_Protected》 |
| 9 | 吸附/拉拔 | - 20N拉拔测试– TP/Inlay（首检） - Inlay 吸附测试– 100%，使用吸盘在Inlay-B5位置进行吸附带起手机确认Inlay无翘起/脱落等异常 |
| 10 | 断差与缝隙标准 | 参照PSCD管控标准：详见《产品外观检验规范标准和品质保证》 |
| No | 功能检验 | 标准 |
| 11 | 检前须知 | 检验前针对按上下音量键进入下一项测试，请注意以下几点： 1. 按下音量键：该测试项Pass 2. 按上音量键：该测试项Fail 3. 在按上、下音量键进入下一项测试之前，会有短暂的停顿状态，不要长按，否则会进入下下测试，并且Pass 或Fail了上一项测试，必须等待。 4. 通常只需短按上、下音量键进入下一项测试。但在Key Test和Headset Info test中，必须长按不超过2秒钟进入下一项测试，因为短按只是在测试按键功能。 5. 在Sequential Test测完后，屏幕上回显示绿色英文的测试结果列表。 6. 插入SIM卡和SD卡时如有阻塞感，切勿强行插入避免造成卡槽损坏。 8. 相机测试需要使用色板进行测试 |
| 12 | 插入mini SIM卡开机 | 1. 拔出SIM卡托插入Mini SIM。 2. 按电源键开机,有轻微震感。 |
| 13 | CQA 测试 CQA Test | 进入CQA测试模式：点击手机屏幕上拨打电话图标，输入“*#*#2486#*#*”, 选Sequence Mode，弹出Confirmation对话框中选择“YES”，出现弹窗提示点击“Allow”进入测试： - Headset/Headset Info: 显示 Headset Detect Status: UNPLUGGED. Headset Has Microphone: NO. Headset Key Press Status: NA； - 插耳机变为: Headset Detect Status: PLUGGED. Has Microphone : YES， Headset Key Press Status: NA. 2）按耳机开关显示KEYCODE_ HEASSET HOOK pressed, 松开显示released； 拔下耳机显示如下:Headset/Headset Info: 显示 Headset Detect Status: UNPLUGGED. Headset Type: No Device. Headset Has Microphone: No. Headset Key Press Status: KEYCODE_ HEADSET HOOK -released 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。（注：需长按下音量键，才可进入下一个测试项，且最长只能按2秒钟） If the test FAIL, press on the volume FAIL;If PASS, then press the volume button to enter the next test.(note: need to long press the volume button, to get to the next test, and the longest only by 2 seconds) - NFC Test: show tag found & EDIT ME 近场通讯测试:然后放入NFC测试工装，主机背面朝向磁卡，显示”TAG FOUND ”&“EDIT ME!”（如图）即为”Pass” - Key/Key Test/Hall test：显示No Key is pressed. 按上音量键显示KEYCODE_VOLUME_ UP Pressed, 松开显示KEYCODE_VOLUME _UP Released。 按下音量键显示KEYCODE_VOLUME DOWN Pressed,松开显示KEYCODE_VOLUME_ DOWN Released，按下音量键。 按电源键，屏幕变暗后返回界面显示KEYCODE_POWER Pressed and Released。 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Hit Volume Up key，Verify correct key code response，Hit Volume Down key，Verify correct key code response，Press power key，Verify display goes dark and comes back with correct key code response，Press and Hold Volume Down key for a one second, and then let go to pass and move to the next test，To fail the test, Press and Hold Volume Up key for a one seconds and then let go Volume Down Key Power Key - Bluetooth/ Bluetooth Scan：出现对话框，点击 Allow, 搜索蓝牙设备，若未收索到蓝牙设备，则此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Select ‘Allow’ to turn on Bluetooth，Verify Bluetooth devices are discovered，Hit Volume Down to pass, Volume Up to fail. - Bluetooth/ Bluetooth Mac：显示蓝牙地址，若未出现蓝牙地址，则此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify Bluetooth Mac address is displayed，Press and Hold Volume Down key to pass and move to the next test，To fail the test, Press and Hold Volume Up key - Audio/Mic Loopback音频测试: 1）依次点击PRIMARY MIC，SECONDARY MIC、TERTIRAY MIC，分别堵住相应的MIC孔，对着手机MIC说“喂喂喂”和“1234”，能从扬声器中听到清晰的回音。所有MIC均插入耳机并用手指堵住相应的MIC孔进行测试。 2）HEADSET MIC测试方法：插入耳机，选择HEADSET MIC, 对着耳机MIC说“喂喂喂”，能从耳机中听到清晰的回音。 3）若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - Press the hole of PRIMARY MIC、SECONDARY MIC、TERTARY MIC, Put phone to ear and talk into MIC, listen for your voice from ‘earpiece’. And every MIC should be pressed by finger. - HEADSET MIC test: insert headset and HEADSET MIC , put phone to ear and talk into MIC, listen for your voice from ‘earpiece’. - Press VOL DOWN key to pass TEST or VOL UP to fail TEST(‘audio/MIC Loopback’) - Ear speaker： 1）BUZZ Test：把手机放到耳边，能听到高低交错的长鸣音，检查是否有异常，如声音偏小，有杂音等。注意：测试两次或以上。 2）再选择Play Harvard speed pattern，把手机放到耳边，能听到一段英文对话，检查是否有异常，如声音偏小，播放过程中有杂音等。. 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 - Listen to BUZZ TEST audio from Ear Speaker, Verify audio is clear, should listening twice. 2) Select PLAY HARVARD SPEECH PATTERN,Listen for media audio from Ear Speaker; Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key,Alert Speaker, Listen to audio file from Alert Speaker,Verify audio is clear;Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - Audio/Loudspeaker： 1) 点击Play Media File –MOTO.OGG, 能听到喇叭播放音乐,且声音清晰无杂音，无卡顿现象。注意：需把整段音乐听完才能进入下一项。 2) 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 1) Listen to audio file from Alert Speaker; Verify audio is clear, should listening the whole music. 2) Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - SIM Card: - 只需检查SIM serial number：SIM卡序列号出现即可，如下图。双卡手机需要测试2张SIM卡。（注：SIM state 可能显示UNKNOWN，视为PASS）。 - Verify SIM STATE: READY and SIM parameters listed,When using 2 SIM cards, verify that the SIM2 parameters are listed, Press and Hold Volume Down key to pass and move to the next test, To fail the test, Press and Hold Volume Up key. - MMC/SD Card self Test: 1) 选择Query MMC current state，检查MMC card 状态为Mounted。(反之显示红色字体:MMC CardUnmounted) 2) 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 1) Verify ‘SD Card Mounted’ message, 2) Press and Hold Volume Down key to pass and move to the next test,To fail the test, Press and Hold Volume Up key - Version版本测试： 1）显示AP Version，BP Version，CQA Test Package Version即可。 2）若不一致，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 1) Verify AP Version, BP Version, and CQATest Package Version are matching what is specified by BR 2) Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - Display显示屏测试：（主要检查显示屏同色点，异色点，划伤，若有其它缺陷，则视为FAIL，除非有限度样本。） 1）低亮度六色块-->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 2）高亮度六色块-->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 3）从黑到灰渐变色->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 4）黑屏->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 5）白屏->若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 6）黑屏白边框。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify ‘Medium Color Bar’ for defects,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key,Verify ‘Bright Color Bar’ for defects and follow step 2) ,Verify ‘Grayscale’ for defects and follow step 2) ,Verify ‘Black’ for defects and follow step 2) ,Verify ‘White with Black Boarder’ for defects and follow step 2) ,Verify ‘Black with White Boarder’ for defects and follow step 2) - Touch Screen触屏测试： 用手指点击白色圆点，沿着白色路径划线，划完后，检查屏幕是否变绿，变绿后将自动进入下一项测试。注意是否有跳线不良现象。 Trace the path with a finger until screen turns green indicating Pass,Test will Pass automatically when tracing is completed successfully,To fail the test, Press and Hold Volume Up key. - Sensor/ Front Light光感测试: 显示绿色英文：Light Sensor：NORMAL 1) 用手或手指遮挡Light sensor（位于手机顶部）, 显示Light Sensor: DARK’. 2) 手或手指移开light sensor后，显示‘Light Sensor: BRIGHT’。 3）若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 1) Verify ‘Front Light’ sensor ,Verify that blocking the light sensor (located at the top of the phone) with a hand or finger causes phone to indicate ‘Light Sensor: Dark’, 2) Verify that if light sensor is not blocked phone indicates ‘Light Sensor Brightest, Brighter, Bright, or Normal’, 3) Press and Hold Volume Down key to pass and move to the next test,To fail the test, Press and Hold Volume Up key - Sensor/proximity距离感应器测试： 用手遮到感应点，显示红色英文“Proximity Status: Enable”， 拿开手, 显示红色英文“Proximity Status: Disable”后自动进入下一项测试,若不出现Enable或Disable或出现其他异常，此次测试FAIL, 则按上音量键表示Fail； Verify ‘Proximity’ sensor,Verify that with nothing near or touching the proximity sensor (located at top of the phone) the phone indicates ‘Status: Disable’ in Red ,Verify that if proximity sensor is touched or blocked with a finger or hand the phone indicates ‘Status : Enable’ in Green and then ‘Status Saturation’ in Blue,Phone automatically switches back to next menu after Proximity test passes,To fail the test, Press and Hold Volume Up key. - Sensor/Magnetometer磁力计测试： 移动手机，X、Y、Z白色数值变化即可。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Magnetometer: Verify that movement of the phone (turning and twisting its orientation) changes displayed numbers,Press and Hold Volume Down key to pass and move to the next test.To fail the test, Press and Hold Volume Up key. - Sensor/Gyroscope陀螺仪测试： 移动手机，X、Y、Z白色数值变化即可。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Gyroscope: Verify that movement of the phone (turning and twisting its orientation) changes the x, y, and z numbers,Press and Hold Volume Down key to pass and move to the next test.To fail the test, Press and Hold Volume Up key. - Sensor/Accelerometer加速度传感器测试： 移动手机，X、Y、Z白色数值变化即可。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Accelerometer : Verify that movement of the phone (turning and twisting its orientation) changes the x, y, and z numbers,Press and Hold Volume Down key to pass and move to the next test.To fail the test, Press and Hold Volume Up key. - WLAN/Scan Networks: 显示网络名称。若未收索到，则此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify WLAN Networks are discovered and Network list appears,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - WLAN:/WiFi Mac： 显示网络地址即可。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。(出现为连续的00也为fail) ‘WiFi Mac’test ,Verify WiFi Mac Address is displayed,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key. - Vibratory马达测试: 按“OFF”有震动，按“ON”震动停止，检查震动是否停止，不能出现震动异音，震动异常等。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Touch On / Off switch to turn vibrator on,Verify vibrator is on,Touch On / Off switch to turn vibrator off,Verify vibrator off,Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key - Battery/Battery Info: 插上充电器，检查电池图标里是否有闪电图标，并检查前4项符合如下标准（粗体）以及后拍M Logo灯是否亮起，--此项检查在检查完其他项目后单独测试。 Battery Voltage >3.000V, Battery Healthy: BATTERY_HEALTHY_GOOD. Battery Plugged: BATTERY_ PLUGGED_AC Battery Present: TRUE 若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 注意：电量大于等于15%为PASS。 1.电量＜50%时，屏幕背景色为黑色。 2.50% ≤电量<60%时，屏幕背景色为红色。 3. 60% ≤电量<80%时，屏幕背景色为黄色。 4.电量≥80%时，屏幕背景色为绿色。 Verify ‘BatteryInfo’ - Battery Level should be higher than 15% - Battery Voltage should be higher than 3.700V - Battery Health should indicate BATTERY_HEALTH_GOOD - Battery Present: TRUE - Press and Hold Volume Down key to pass and move to the next test. To fail the test, Press and Hold Volume Up key NOTE: - Battery Level will be in RED for results < 50% - Background color will change for results >50% - Background is ORANGE for <60% - Background is YELLOW for <80% - Background is GREEN for > 80% M Logo灯亮起 M Logo灯没亮 - 屏下指纹测试： CQA test/选择sensor/选择fingerprint self test/用手指点击屏幕中间绿色按钮进行测试(不带指套的手指)，测试成功提示Succeed，按下音量下键进入下一项。 Place finger on the green Finger Print Sensor, Verify test result is “Succeed”,Press and Hold Volume Down key to pass and move to the next test;To fail the test, Press and Hold Volume Up key. - Camera多媒体测试： - 进入前摄像头拍照界面，点击屏幕任意一处进行拍照，注意听是否有“咔嚓”一声响（注：若没听到“咔擦”声，请注意音量是否调至最大）。拍照时，检查是否有异常，如模糊，异色，缺划等，若不能拍照，无咔嚓声存在其他缺陷，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify the front camera‘s preview is normal, no contrast color, light line etc. on the screen. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - 进入前摄像头录像状态，点击“start capture”开始录像，对着手机说‘喂喂喂和1234’，最多录10秒后，点击“stop capture”停止录像。点击“start playback”播放录像，查看录像是否清晰，声音是否正常，点击“stop playback”停止播放。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Enter front camera video interface, press the ‘Start Capture’ button to start video capture; after no more than 10 seconds press the ‘Stop capture’ button; Press the ‘Start Playback’ button and confirm playback works; Press ‘Stop Playback” button and then stops. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. As picture 2. - 进入后摄像头①拍照状态，点击屏幕任意一处进行拍照，注意听是否有“咔嚓”一声响（注：若没听到“咔擦”声，请注意音量是否调至最大）且闪光灯闪亮。拍照时，检查是否有异常，如模糊，异色，缺划等，若不能拍照，无咔嚓声存在其他缺陷，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify the rear camera①‘s preview is normal, no contrast color, light line etc. on the screen. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - 进入后摄像头①录像状态，点击“start capture”开始录像，对着手机说‘喂喂喂和1234’，最多录10秒后，点击“stop capture”停止录像。点击“start playback”播放录像，查看录像是否清晰，声音是否正常，点击“stop playback”停止播放。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Enter rear camera① video interface, press the ‘Start Capture’ button to start video capture; after no more than 10 seconds press the ‘Stop capture’ button; Press the ‘Start Playback’ button and confirm playback works; Press ‘Stop Playback” button and then stops. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - 进入后摄像头②拍照状态，点击屏幕任意一处进行拍照，注意听是否有“咔嚓”一声响（注：若没听到“咔擦”声，请注意音量是否调至最大）。拍照时，检查是否有异常，如模糊，异色，缺划等，若不能拍照，无咔嚓声存在其他缺陷，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify the rear camera②‘s preview is normal, no contrast color, light line etc. on the screen. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - 进入后摄像头②录像状态，点击“start capture”开始录像，对着手机说‘喂喂喂和1234’，最多录10秒后，点击“stop capture”停止录像。点击“start playback”播放录像，查看录像是否清晰，声音是否正常，点击“stop playback”停止播放。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Enter rear camera② video interface, press the ‘Start Capture’ button to start video capture; after no more than 10 seconds press the ‘Stop capture’ button; Press the ‘Start Playback’ button and confirm playback works; Press ‘Stop Playback” button and then stops. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - 进入后摄像头③拍照状态，点击屏幕任意一处进行拍照，注意听是否有“咔嚓”一声响（注：若没听到“咔擦”声，请注意音量是否调至最大）且闪光灯闪亮。拍照时，检查是否有异常，如模糊，异色，缺划等，若不能拍照，无咔嚓声存在其他缺陷，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify the rear camera③‘s preview is normal, no contrast color, light line etc. on the screen. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - 进入后摄像头③录像状态，点击“start capture”开始录像，对着手机说‘喂喂喂和1234’，最少录10秒后，点击“stop capture”停止录像。点击“start playback”播放录像，查看录像是否清晰，声音是否正常，点击“stop playback”停止播放。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Enter rear camera③ video interface, press the ‘Start Capture’ button to start video capture; after no more than 10 seconds press the ‘Stop capture’ button; Press the ‘Start Playback’ button and confirm playback works; Press ‘Stop Playback” button and then stops. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - 进入后摄像头④拍照状态，点击屏幕任意一处进行拍照，注意听是否有“咔嚓”一声响（注：若没听到“咔擦”声，请注意音量是否调至最大）且闪光灯闪亮。拍照时，检查是否有异常，如模糊，异色，缺划等，若不能拍照，无咔嚓声存在其他缺陷，则此次测试FAIL, 按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Verify the rear camera③‘s preview is normal, no contrast color, light line etc. on the screen. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. 10）进入后摄像头④录像状态，点击“start capture”开始录像，对着手机说‘喂喂喂和1234’，最少录10秒后，点击“stop capture”停止录像。点击“start playback”播放录像，查看录像是否清晰，声音是否正常，点击“stop playback”停止播放。若此次测试FAIL, 则按上音量键表示Fail；若PASS，则按下音量键进入下一项测试。 Enter rear camera④ video interface, press the ‘Start Capture’ button to start video capture; after no more than 10 seconds press the ‘Stop capture’ button; Press the ‘Start Playback’ button and confirm playback works; Press ‘Stop Playback” button and then stops. Press VOL DOWN key to pass TEST or VOL UP to fail TEST. - Capsense test/电容感应测试： 点击capsense 进入测试，将手掌贴近感应器对应位置（CH0/CH1/CH2/CH3），状态显示由“absence”转为“presence”,则测试pass， 如状态显示“absence ”未变化，则测试fail. Press capsense button to enter test , push hands close to sensor which is on the corresponding position ( CH0/CH1/CH2/CH3). if the status turn form “absence” to “presence”, the test is pass ;if the status of “absence ”can’t change ,the test is fail - USB Host mode/串口模式测试： 使用OTG数据线+U盘进行插拔测试，正反各一次，如测试结果为绿色字体,则测试结果为pass,如返回值为”NO”，则测试结果fail。 Verify OTG and U Disk insertion and extraction,both two sides,if the status turn to green, the test is pass. if the status can’t change ,the test is fail. - Type-C reverse test/Type-C 翻转测试 使用Type-C 数据线正反插拔USB各一次，如测试结果为”YES&OK”,则测试结果为pass,如返回值为”NO”，则测试结果fail. Verify the Type-C usb insertion and extraction, both two sides, if the status turn to green and Yes/ok, the test is pass. if the status can’t change ,the test is fail. |
| 13 | 关机 Power off | 按关机键，屏幕显示Power off, 点击OK，关机，有振感；取出SIM卡即可。 Press the Power Key, the screen shows the Power off, click OK, shutdown, a sense of vibration;Take out the SIM card and SD card. |

- Relevant documents/相关文件

| 序号 | 文件编号 | 文件名称 |
| --- | --- | --- |
| 1 | WHM-QA-Quattro-001 | Motorola Quattro PSCD产品外观检验规范标准和品质保证 |
| 2 | WHM-QA-GN-101 | 手机和移动设备外观检验规范 |

- Relevant records/相关记录

| 序号 | 记录编号 | 文件名称 | 责任部门 | 保存方式 | 保存期限 |
| --- | --- | --- | --- | --- | --- |
| 1 | WHM-QA-QR-134 | WHPQuattro L10首检记录表 | CQA | 纸档 | 2年 |